Development Applications – Standard Conditions

Health Premises Generally	1

Industrial Premises	2

Wastewater Disposal	3

Sanitary Conveniences	5

Noise	5

Contaminated Sites	6

Ancillary Accommodation	7

Note: The use of conditions and footnotes can vary between Council’s. It is best to confirm with your Planning Department what will be accepted as a condition rather than a footnote; the below is a guide of frequently used conditions and footnotes compiled from various local government authorities.

# Health Premises Generally

Conditions:

- The provision of a refuse storage enclosure being provided, of an adequate size to contain all bins and oil containers used by the premises and be constructed with four walls, a graded and sealed floor, provided with a tap connected to an adequate supply of water and provided with a floor waste connected to the sewer.

- The premises is required to be registered with the Local Government in accordance with the Food Act 2008; the applicant is required to submit an Application for Registration of a Food Business accompanied with the appropriate registration fee a minimum of two weeks prior to commencing operation.

Footnotes:

- The premises has not been approved as a lodging house in accordance with the Health Act 1911; subsequently no more than 5 lodgers or guests (exclusive of the family) may reside on the premises at the same time.

- Compliance with Food Act 2008, the Food Regulations 2009, the Australia New Zealand Food Standards Code and the Australian Standard AS4674:2004 Design, construction and fit-out of food premises. Detailed fit-out plans, including elevations, are to be submitted with the building licence application showing all fixtures, fittings and finishes.

- Compliance with the Health (Public Buildings) Regulations 1992; detailed plans are to be submitted with the building licence application.

- Compliance with Hairdressing Establishment Regulations 1972; detailed fit out plans are to be provided with the building licence application.

- Compliance with Health (Skin Penetration Procedure) Regulations 1998 and the Code of Practice for Skin Penetration Procedures; detailed fit out plans are to be provided with the building licence application.

# Industrial Premises

Conditions:

- All external openings to the warehouse are to be provided with bunding at a height of 25mm or alternatively install cut-off drains (spoon drains) to the floor at all external openings which are connected internally to a below ground impervious sump at a minimum size of 600mm by 600mm provided with a pump out point.

- All hydrocarbons, oils, paints and chemical liquids which are stored onsite and which are greater than 50 litres in quantity, shall be stored within an area which;

  - is designed to contain at least 25% of the total volume of the vessels stored within the compound;

  - is provided with a flooring that is chemically resistant to the chemicals stored within it and is of hardstand construction; and

  - the floor to be graded to an impervious sump and must prevent the entry and collection of rainwater, unless the hydrocarbons, oils, paints and chemical liquids are stored in reinforced intermediate bulk containers.

- All powder coating must be carried out within a powder coating booth or room which has;

  - four walls and a roof;

  - a sealed impervious floor

  - bunding provided at all external door openings; and

  - is ventilated in accordance with AS/NZS 3754 - 1990.

- All abrasive blasting must be undertaken within a booth that has;

  - four walls and a roof;

  - a sealed impervious floor;

  - bunding provided at all external door openings; and

  - is completely sealed at all times abrasive blasting is occurring.

- All spray painting must be carried out within a spray booth or room which has

  - four walls and a roof;

  - a sealed impervious floor;

  - bunding provided at all external door openings; and

  - is ventilated in accordance with AS/NZS 4114.1:1995.

- The construction and installation of the spray booth is to comply with AS 4114:2003 Parts 1 and 2, Worksafe and the Department of Environment and Conservation’ s “Code of Practice for Automotive Spray Painting.

- Exhaust stack of spray painting booth is to be greater than 3 metres above the nearest ridge line within 15 metres.

Footnotes:

- The maintenance and cleaning of vehicles and any other equipment or plant must be carried out in hardstand, bunded and drained areas from where contaminants cannot be released into the environment, any storm water soakwells, the roadside and gutter or storm water drainage system.

- No industrial wastewater is to be discharged into the storm water system.

- Approval may be required from the Department of Mines and Petroleum for the storage of dangerous goods.

- Occupants are to liaise with the Department of Environment and Conservation regarding the registration of activities and processes.

- This development has not been approved for use as a public building, food premises, food storage area, factory, workshop or any other industrial use; these operations require a change of use approval with the submission of a new development application.

- Any liquid chemicals, oils, detergents or other liquid products are to be stored within bunded areas of sufficient size to contain any spillage.

# Wastewater Disposal

Conditions:

- Development to be connected to the Minister’s Sewer, to the satisfaction of the Water Corporation.

- Both the existing dwelling and the proposed new dwellings are to be connected to the Minister’s Sewer.

- A geotechnical report is to be submitted detailing site conditions, particularly in respect to soil and groundwater to determine adequate wastewater disposal can be achieved. The report should consider groundwater clearance, quantity, soil permeability and type, nutrient loading and location and size of recommended wastewater treatment systems.

- Any existing on-site effluent disposal system(s) including all tanks and pipes and associated drainage systems (soakwells or leach drains) are to be decommissioned and removed. A statutory declaration must be provided to the Local Government Authority confirming completion of these works.

- All wastewater produced by the activities onsite must be disposed of to the Sewer, with approval of the Water Corporation of Western Australia.

- All kennels shall be connected to an effluent disposal system, constructed and installed to the satisfaction of the Local Government Environmental Health Officer.

- All bypass/overflow wastewater from commercial oil separator/wastewater treatment unit shall be discharged to sewer in accordance with the Water Corporations Acceptance Criteria for Industrial Waste. OR

- All bypass/overflow wastewater from commercial oil separator/wastewater treatment shall be discharged to an approved onsite effluent system in accordance with the Water Corporations Acceptance Criteria for Industrial Waste and to the satisfaction of the Local Government Health Department.

- Wastewater discharge from the wash bay facility shall be tested post treatment, on a minimum 6 monthly basis, by a NATA accredited laboratory and test results must be kept for a period of no less than 5 years. Treated wastewater shall be analyzed for pH, Total petroleum hydrocarbons, Biological oxygen demand, Suspended solids, Total dissolved solids.

- All washing of vehicles and equipment must be undertaken in an approved wash bay connected to sewer, or if unavailable, connected to an aerobic treatment unit (ATU).

- An “Application to Construct or Install an Apparatus for the Treatment of Sewage” form is to be submitted with the building licence to provide details for the effluent disposal system. The system is to be adequate to treat waste water based on the maximum number of staff/occupants onsite at any one time and is to be in accordance with the Health (Treatment of Sewage and Disposal of Effluent and Liquid Waste) Regulations 1974.

- Provide documentation with the building licence regarding the anticipated volume of waste water and size of the existing waste water disposal system/s. Should the existing effluent disposal system/s not be sufficient, an effluent disposal application is required to be submitted with the building licence for an extension to the system.

- All onsite effluent disposal systems, including all tanks, pipes and associated drainage systems (soak wells or leach drains) are to be decommissioned, removed, filled with clean sand and compacted in accordance with the Health (Treatment of Sewerage and Disposal of Effluent and Liquid Waste) Regulations 1974. A statutory declaration is required to be completed by the person responsible for undertaking the works to verify decommissioning and removal of the effluent disposal system. The wash down bay is to be bunded to a minimum height of 75mm and connected to sewer with all wastewater to be disposed of according to the Water Corporations requirements.

- Industrial wastewater from wash bays is required to be treated via a commercial oil separator/waste water treatment unit in a closed loop water recycling system.

- The wash down bay is to be bunded to a minimum height of 75mm and connected to sewer with all wastewater to be disposed of according to the Water Corporations requirements.

- The following specifications are required for the wash down bay and details are to be included with the building licence application;

- The wash down bay is to be bunded to a minimum height of 75mm;

- Provide documentation to confirm that wastewater going to the leach drain can be treated to the standard of; hydrocarbons <30ppm, suspended solids <1500ppm and pH between 6-9;

- Provide details on the expected daily volume of wastewater being disposed of to the leach drain;

- Provide specifications of the treatment system being installed;

- Degreasers and detergents are to be of the “quick break ” type only, details to be provided;

- Provide an industrial waste sampling point and provide details on the procedures to be implemented for the ongoing maintenance of the system eg testing of wastewater for compliance with recommended parameters, cleaning out of silt pits etc;

- Provide details regarding the use of the wash down bay including the type and frequency of equipment/vehicles being washed down;

- Provide confirmation that the capacity of the silt pits allows for adequate retention time to enable settling of solids;

- If wash down bay will not be roofed, information is to be provided to verify that it can cope with a 1 in 10 year storm event; and

- Provide details of the petrol and water separator to be used.

Footnotes:

- Wastewater produced from washing of vehicles and vehicle maintenance must not be discharged onsite or into any storm water drain or soakwell.

- The development is to maintain the appropriate setbacks to any existing effluent disposal system/s, being 1.8m from a leach drain and 1.2m from a septic tank.

- The applicant is advised that should the on-site waste water system fail to cope with any additional loading that may result as a consequence of this development the Local Government may require the system to be upgraded.

# Sanitary Conveniences

Conditions:

- The water closet shall open to an airlock with a floor area of at least 1.85m2 complying with the Sewerage (Lighting, Ventilation and Construction) Regulations 1971; or otherwise made to comply with the aforementioned regulations.

- The sanitary convenience shall be mechanically ventilated in accordance with the Sewerage (Lighting, Ventilation and Construction) Regulations 1971 and the exhaust air flued to open air.

Nuisances

Conditions:

- Stockpiling or blending of manures or compost shall not occur on the premises.

- No dust or particulates shall be discharged beyond the boundaries of the premises.

- Provide a written report, prepared by an independent odour expert which;

  - confirms that the pollution control equipment installed is the best available technology relevant to the industry and that;

  - offensive odours are not adversely impacting nearby residential, commercial and industrial land. The report should be submitted to the Local Government Authority within 3 months of operations commencing.

- Provide an Environmental Consultants Report with regard to odour control; if the potential for offensive odours are identified to occur provide an Odour Management Plan for the control practices to prevent odour nuisances from occurring.

Footnotes:

- The applicant is reminded of their obligations to comply with the Environmental Protection Amendment Regulations (No 2) 2000, whereby vegetation to be removed shall not be burnt on site.

- The applicant is reminded of their obligations to comply with the “Land development sites and impacts on air quality: a guideline for the prevention of dust and smoke pollution from land development sites in Western Australia”, prepared by the Department of Environment & Conservation.

# Noise

Conditions:

- Prior to the issue of a Certificate of Classification, an Acoustic Report is to be prepared by a suitably qualified Acoustic Consultant and be submitted to the Local Government Authority and must demonstrate that all mechanical services associated with the intended use comply with the Environmental Protection (Noise) Regulations 1997.

- Prior to the issue of a Building Licence, an Acoustic report is to be prepared by a suitably qualified Acoustic Consultant and be submitted to the Local Government Authority to the satisfaction of the Principal Environmental Health Officer and must demonstrate that the noise levels received inside the dwellings will comply with Australian Standard 2021, when received from nearby industrial and commercial premises.

- Prior to the issue of a Building Licence, an Acoustic report is to be prepared by a suitably qualified Acoustic Consultant and be submitted to the Local Government Authority and must demonstrate that all mechanical services associated with the intended use, are capable of complying with the Environmental Protection (Noise) Regulations 1997, when the noise is received at any noise sensitive premises.

- The recommendations contained within the Acoustic Report, dated [*insert], produced by [*insert] must be detailed on plans submitted for a Building Licence and implemented.

- Having regard to State Planning Policy 5.4 - Road and Rail Transport Noise and Freight Considerations in Land Use Planning, the applicant is required to engage an appropriately qualified acoustic consultant (such as a member of the Australian Acoustical Society or the Association of Australian Acoustical Consultants) to undertake a detailed noise assessment in accordance with the Implementation Guidelines for the above policy, to the satisfaction of the Local Government prior to commencing the development . Where such an assessment finds the proposal fails to meet the objectives of the policy for either indoor or outdoor noise levels, the proponent must submit a noise mitigation plan for approval of the Local Government prior to commencing the development. The approved noise mitigation plan will subsequently form part of the development approval for this proposal.

Footnotes:

- All dwellings on the premises must meet the requirements of Australian Standard 2021 in relation to interior noise levels.

# Contaminated Sites

Conditions:

- Prior to any construction activities or development of the land, investigation for the presence of acid sulfate soils shall be carried out in accordance with the guidelines established by the Department of Environment and Conservation, to the satisfaction of the Local Government Authority and should such investigation confirm the presence of acid sulfate soils, an Acid Sulfate Soils Management Plan is to be prepared and submitted to and approved by the Local Government Authority before any site works are commenced.

- If an acid sulfate soils management plan has been approved in accordance with condition [insert condition No.] then accordingly all site works shall be carried out in accordance with the provisions of that Acid Sulfate Soils Management Plan.

- Prior to any construction activities commencing or development of the land, a preliminary investigation for soil and groundwater contamination must be completed, in accordance with Department of Environment and Conservation guidelines and a copy of the report submitted to the Local Government Authority.

Footnotes:

- All material known or suspected to contain asbestos is to be removed and disposed of in accordance with the Environmental protection (Controlled Waste) Regulations 2004 and the Health (Asbestos) Regulations 1993.

# Ancillary Accommodation

Conditions:

- The ancillary accommodation is to be connected to sewer; OR

- The ancillary accommodation must be connected to a new suitable on-site effluent disposal system to the satisfaction of the Local Government Environmental Health Officer; OR

- The ancillary accommodation must be connected to the existing effluent disposal system.

- The ancillary accommodation application is refused as the development is greater than 60m2 and does not comply with the State Sewerage Policy with regards to the property lot size and development.